Motion Control - User Manual

By

Devant Yadav, Akuete Akwei, Charles Lejtman

Ultrasonic Device for Motion Control

This device’s purpose is to enable a user to control certain features of VLC media player and a maze game on a computer without using a mouse or keyboard. The device uses ultrasonic sensors to capture hand movements by computing and mapping varying distances generated by back and forth hand motion into keyboard inputs.

- Proper Placement of hands

To control various programs using the device, it is important to have hands in the correct position in front of the sensors. To ensure the help the user identify where to place hands, LEDs have been added to the device.

- A Green LED will light up if the user places his/her right hand within 13 to 17 cm from the right sensor.

- A Red LED will light up if the user places his/her left hand within 13 to 17 cm from the left sensor.

- A Yellow LED will light up if the user places both hands within 30 to 50 cm in  front of both sensors. The led can also be triggered if the user leans their head in the same distance from the sensors.

- Controlling VLC media player

In order to control VLC media player, the user needs to do the following:

- Volume up :

  - When red LED turns on, move hand closer to the left sensor.

- Volume down:

  - When red LED turns on, move hand further from the left sensor.

- Forward:

  - When green LED turns on, move hand further from the right sensor.

- Rewind:

  - When green LED turns on, move hand closer to the left sensor.

- Play/Pause:

  - Yellow LED triggered by placing both hands being within 30 to 50 cm of sensors, or by user’s head being within the same distance

- Playing the maze game

In order to play the maze game, the user needs to do the following:

- Forward :

  - When red LED turns on, move hand further from the left sensor.

- Backward :

  - When red LED turns on, move hand closer to the left sensor.

- Left:

  - When green LED turns on, move hand further from the right sensor.

- Right :

  - When green LED turns on, move hand closer to the right sensor.